# Gen 3 Sienna Vehicle Delivery Generic Checklist … S.Mahler 150615

## What to bring with you

- Tire pressure gauge

- 4-6 CDs with your favorite music (depending on the size of your CD-changer)

- DVD, Blu-Ray & SD card with your favorite movie and/or music

- Low-power 110V device (100 watts max) with 2-prong plug (to test power inverter)

- Car cell phone charger (to test 12V outlets)

- Flashlight (to check fluid levels)

- Insurance card and driver’s license

- Cash, valid check, gold, silver, arm, leg

## If Trading-In

- Title for the trade-in car (and all other required documents)

- All sets of keys + valet key, all key fob transmitters

- User Manual(s)

- PIN codes for keys, car stereo, Navigation system

- If Navigation on the trade-in, reset personal addresses and previous destinations

- If OnStar, XM or Sirius equipped, stop the service

- Reset Homelink remote control, if necessary

- Remove all personal belongings, check behind sun visors, under the seats, in storage compartments, etc.

## EXTERIOR

- Remote lock/panic/unlock using key fob remote

- Start engine and listen for noises while outside the vehicle

- Turn on headlights, fog lights and emergency blinkers

- Pull the hood release lever (under the dash)

### Front

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Undamaged | Body panels for dents, scratches, spots, paint quality | Y |  |  |
| Undamaged | Front bumper fascia | Y |  |  |
| Undamaged | Windshield glass and frame | Y |  |  |
| Undamaged | Radar cruise control sensor if equipped | Y |  |  |
| Alignment | Body panels and front fascia seams even | Y |  |  |
| Alignment | Hood with the body, head lights and front fascia | Y |  |  |
| Operational | Antenna (if installed) | Y |  |  |
| Operational | Headlights | Y |  |  |
| Operational | Fog lights | Y |  |  |
| Operational | Side marker lights | Y |  |  |
| Secure | Front parking sonar sensors |  |  |  |
| Secure | Cowling (black plastic strip) is glued to the bottom of the windshield glass |  |  |  |
| Secure | Wiper arms and blade inserts |  |  |  |
| Installed | Front license plate bracket (if required) |  |  |  |
| Verify | Windshield glass VIN etch (if ordered) | Y |  |  |
| Verify | Any required state inspection sticker(s) [LA 2 Yr.] | Y |  |  |

### Under hood

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Operational | Open the hood | Y |  |  |
| Operational | Hood prop rod | Y |  |  |
| Undamaged | Battery, wiring, hoses and drive belts | Y |  |  |
| Level | Antifreeze | Y |  |  |
| Level | Windshield washer fluid | Y |  |  |
| Level | Brake fluid | Y |  |  |
| Level | Power steering fluid | Y |  |  |
| Level | Engine oil |  |  |  |
| Level | Transmission fluid |  |  |  |
| Installed | Spare fuses in fuse block |  |  |  |
| Verify | No fluid leaks | Y |  |  |
| Operational | Close the hood | Y |  |  |

### Driver's Side

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Undamaged | Body panels for dents, scratches, spots, paint quality | Y |  |  |
| Undamaged | Door seals | Y |  |  |
| Undamaged | Mirror surface | Y |  |  |
| Undamaged | Window glass | Y |  |  |
| Undamaged | Window seals |  |  |  |
| Undamaged | Wheel rims and tires | Y |  |  |
| Undamaged | Running boards (if installed) | Y |  |  |
| Secure | Running boards (if installed) | Y |  |  |
| Secure | Body side molding (if installed) | Y |  |  |
| Secure | Mud flaps (if installed) | Y |  |  |
| Alignment | Body panel seams even | Y |  |  |
| Operational | Fold/unfold manual mirror {unless power folding} | Y |  |  |
| Operational | Mirror turn signals / heat / BSM | Y |  |  |
| Operational | Mirror puddle lamps / auto-dim |  |  |  |
| Operational | Open/close sliding door by key remote | Y |  |  |
| Operational | Open/close sliding door by pulling the handle | Y |  |  |
| Operational | Power sliding door reverses direction when there is an obstruction | Y |  |  |
| Operational | Sliding door courtesy light | Y |  |  |
|  | Open driver's door and disable power doors | Y |  |  |
| Operational | Both sliding doors do not open electrically | Y |  |  |
| Operational | Open/close sliding door manually | Y |  |  |
| Operational | Lower the window on the sliding door |  |  |  |
| Operational | Sliding door does not open fully, shut door |  |  |  |
| Operational | Raise the window on the sliding door |  |  |  |
|  | Enable power sliding doors | Y |  |  |
| Operational | Open gas filler cover using the remote release | Y |  |  |
| Operational | Sliding door does not open | Y |  |  |
| Installed | Tire valve stem caps |  |  |  |
| Installed | Wheel covers (if required) |  |  |  |
| Verify | Tire pressure (35psi +/- 2psi) [best at 35.5] |  |  |  |
| Verify | Window glass VIN etch (if required) | Y |  |  |
| Verify | Gas cap can be stowed on fuel filler door |  |  |  |

### Rear

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Undamaged | Rear door for dents, scratches, spots, paint quality | Y |  |  |
| Undamaged | Rear bumper fascia | Y |  |  |
| Undamaged | Door seals |  |  |  |
| Undamaged | Window glass | Y |  |  |
| Secure | Rear spoiler | Y |  |  |
| Secure | Rear parking sonar sensors |  |  |  |
| Secure | Rear wiper arm and blade insert |  |  |  |
| Alignment | Rear door seams even | Y |  |  |
| Operational | Tail and license plate lights | Y |  |  |
| Operational | Open/close power rear door by key remote | Y |  |  |
| Operational | Open/close power rear door by lifting the handle/pushing button | Y |  |  |
| Operational | Power rear door reverses direction when there is an obstruction | Y |  |  |
|  | Open driver's door and disable power rear door | Y |  |  |
| Operational | Power rear door does not open electrically | Y |  |  |
| Operational | Open/close power rear door manually | Y |  |  |
|  | Enable power rear door and open | Y |  |  |
| Operational | 3rd Row Seats Up/Down Individual/Simultaneous | Y |  |  |
| Installed | Towing receiver hitch cap (if installed) |  |  |  |
| Verify | Window glass VIN etch (if required) | Y |  |  |

### Passenger Side

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Undamaged | Body panels for dents, scratches, spots, paint quality | Y |  |  |
| Undamaged | Door seals | Y |  |  |
| Undamaged | Mirror surface | Y |  |  |
| Undamaged | Window glass | Y |  |  |
| Undamaged | Window seals |  |  |  |
| Undamaged | Wheel rims and tires | Y |  |  |
| Undamaged | Running boards (if installed) | Y |  |  |
| Secure | Running boards (if installed) | Y |  |  |
| Secure | Mud flaps (if installed) | Y |  |  |
| Secure | Body side molding | Y |  |  |
| Alignment | Body panel seams even | Y |  |  |
| Verify | Tire pressure (door sticker plus 0.5 psi) | Y |  |  |
| Verify | Window glass VIN etch (if required) | Y |  |  |
| Installed | Wheel covers (if required) |  |  |  |
| Operational | Fold/unfold manual mirror {unless power folding} | Y |  |  |
| Operational | Mirror turn signals / heat / auto-dim / BSM / puddle | Y |  |  |
| Operational | Open/close sliding door by key remote | Y |  |  |
| Operational | Open/close sliding door by pulling the handle | Y |  |  |
| Operational | Power sliding doors reverses direction when there is an obstruction | Y |  |  |
| Operational | Sliding door courtesy light | Y |  |  |
|  | Open driver's door and disable power doors | Y |  |  |
| Operational | Both sliding doors do not open electrically | Y |  |  |
| Operational | Open/close sliding door manually | Y |  |  |
|  | Lower the window on the sliding door | Y |  |  |
| Verify | Sliding door does not open fully, shut door | Y |  |  |
|  | Raise the window on the sliding door | Y |  |  |
|  | Enable power sliding doors | Y |  |  |

### Top (open doors and stand on threshold)

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Undamaged | Roof panel for dents, scratches, spots, paint quality | Y |  |  |
| Undamaged | Sunroof / Moonroof glass (none, one or both) | Y |  |  |
| Undamaged | Sunroof / Moonroof seal (none, one or both) | Y |  |  |
| Secure | Roof rack rails | Y |  |  |
| Installed | Roof rack crossbars (if equipped) | Y |  |  |

### Underneath

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Undamaged | CV joints, drive shafts | Y |  |  |
| Undamaged | Exhaust system | Y |  |  |
| Undamaged | Shocks and springs | Y |  |  |
| Undamaged | Ball joints, tie rods, bushings | Y |  |  |
| Undamaged | Brakes | Y |  |  |
| Installed | Rubber body plugs (4) |  |  |  |
| Installed | Spare tire (FWD only) | Y |  |  |
| Verify | No fluid leakage | Y |  |  |
| Verify | Nothing dangling | Y |  |  |

## INTERIOR

### Driver Seat

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Undamaged | Seat upholstery | Y |  | Check no fabric/leather creases or other anomalies |
| Undamaged | Carpet, lower panels | Y |  |  |
| Undamaged | Floor mat and both mat hooks | Y |  |  |
| Undamaged | Windshield glass optical defects | Y |  |  |
| Undamaged | Door panel | Y |  |  |
| Alignment | Dash and door panel seams | Y |  |  |
| Operational | Power Mirror adjustment and if equipped Folding | Y |  |  |
| Operational | Seat power adjustments | Y |  |  |
| Operational | Seat power reclining | Y |  |  |
| Operational | Seat power lumbar support | Y |  |  |
| Operational | Seat heat |  |  |  |
| Operational | Seat belt extension and retraction | Y |  |  |
| Operational | Overhead console sunroof lift/open/close | Y |  |  |
| Operational | Overhead console door controls | Y |  |  |
| Operational | Master light switch turns off all interior lights | Y |  |  |
| Operational | Steering wheel tilt/telescope adjustments | Y |  |  |
| Operational | Horn, instrument panel, gauges | Y |  |  |
| Operational | Flash high beams | Y |  |  |
| Operational | Turn signals | Y |  |  |
| Operational | Front/rear windshield washers |  |  |  |
| Operational | Set/verify both memory positions |  |  |  |
| Operational | A/C controls (all 3 zones from the front panel) |  |  |  |
| Operational | Radio (AM/FM/XM/HD), including steering wheel-mounted controls |  |  |  |
| Operational | Open/close for access to CD changer |  |  |  |
| Operational | CD changer loads/unloads disks and plays them |  |  |  |
| Operational | Rearview camera (wide angle view too, if equipped) |  |  |  |
| Operational | Navigation voice guidance (a few simple commands) |  |  |  |
| Operational | Sun visor motions and mirror lights |  |  |  |
| Operational | All dash lights | Y |  |  |
| Operational | Ignition switch light |  |  |  |
| Operational | Steering wheel mounted lights (no flicker) |  |  |  |
| Operational | Cruise control stalk light (no flicker) |  |  | CC stalk lights can flicker, visible at night |
| Operational | Map lights/dome lights |  |  |  |
| Operational | Door courtesy light |  |  |  |
| Operational | Both 12VDC outlets |  |  |  |
| Operational | 120VAC switch |  |  |  |
| Operational | Side mirror adjustment and fold |  |  |  |
| Operational | Automatic headlights (cover dash sensor in daytime) |  |  |  |
| Operational | HID level adjust | Y |  |  |
| Operational | Auto down/up all windows |  |  |  |
| Operational | Power door lock/unlock |  |  |  |
| Operational | All door and rear vent window controls |  |  |  |
| Operational | Heated mirrors |  |  |  |
| Operational | Start engine with all keys | Y |  |  |
| Operational | Bluetooth connectivity |  |  |  |
| Alignment | Extractable cup holder |  |  |  |
| Alignment | Center dash console bins |  |  |  |
| Installed | Center console |  |  |  |
| Installed | n/a |  |  |  |
| Installed | Shift-lock override button cover |  |  |  |
| Verify | Latest navigation SD version (eject SD - the version is on the label). |  |  |  |
| Verify | Enough gas in the tank to make it to the gas station |  |  |  |
| Verify | Valet key will open/close door and start the car |  |  |  |
| Verify | In the dark, make sure all buttons illuminate |  |  |  |
| Verify | Navigation system stored destinations empty – use “Delete All” to clear any present |  |  |  |
| Verify | While all doors are closed, open and close the driver's door, make sure no excessive force is required to close it (air tightness of the car) |  |  |  |
| Verify | Pre-tensioners on the seatbelts | Y |  |  |
| Verify | Belt assembly to be correct (proper location and orientation of the clip) | Y |  |  |
| Verify | SRS Knee Bag vinyl cover removed | Y |  | Some have clear protective |
| Verify | Clock is set |  |  |  |
| Verify | Custom body electronics system programming requested was done correctly |  |  |  |
| Verify | Odometer has few nominal miles | Y |  | Reduce price for demo |

### Passenger Seat

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Undamaged | Seat upholstery | Y |  | Check no fabric/leather creases or other anomalies |
| Undamaged | Carpet, door panel | Y |  |  |
| Undamaged | Floor mat | Y |  |  |
| Undamaged | Windshield glass optical defects | Y |  |  |
| Undamaged | Dash and door panel seams | Y |  |  |
| Alignment | Panel seams | Y |  |  |
| Alignment | Both glove compartments | Y |  |  |
| Operational | Power door lock/unlock | Y |  |  |
| Operational | Power window | Y |  |  |
| Operational | Sun visor motions and mirror lights | Y |  |  |
| Operational | Seat power adjustments | Y |  |  |
| Operational | Seat reclining / fore and aft motion | Y |  |  |
| Operational | Seat heat | Y |  |  |
| Operational | Seat belt extension and retraction |  |  |  |
| Operational | Under seat storage tray (if installed) |  |  |  |
| Operational | Purse hook |  |  |  |
| Operational | Upper glove box |  |  |  |
| Operational | Lower glove box locks w/ 2 main keys not valet key |  |  |  |
| Verify | Pre-tensioners on the seatbelts |  |  |  |
| Verify | Belt assembly to be correct (proper location and orientation of the clip) |  |  |  |
| Verify | Valet key will not lock/unlock lower glove box |  |  |  |

### Second Row

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Undamaged | Seat upholstery | Y |  | Check no fabric/leather creases or other anomalies |
| Undamaged | Carpet, door panels | Y |  |  |
| Undamaged | Floor mat | Y |  |  |
| Undamaged | Pockets on the back of the front seats | Y |  |  |
| Undamaged | Door panel seams | Y |  |  |
| Alignment | Panel seams | Y |  |  |
| Operational | Door lock/unlock | Y |  |  |
| Operational | Child safety locks | Y |  |  |
| Operational | Both sliding doors open/close by inside switch |  |  |  |
| Operational | Overhead console moon roof open/close/tilt | Y |  |  |
| Operational | Power windows | Y |  |  |
| Operational | Seats tumble, adjust – including footrest if equipped | Y |  |  |
| Operational | Seat belt extension and retraction | Y |  |  |
| Operational | Sunshades |  |  |  |
| Operational | 110V outlet |  |  |  |
| Operational | Overhead lights | Y |  |  |
| Operational | Overhead vent assemblies |  |  |  |
| Operational | Rear A/C controls (on overhead) |  |  |  |
| Operational | Front center console drawer |  |  |  |
| Operational | Insert DVD/BR/SD card into the RES disk reader and start the movie | Y |  |  |
| Operational | RES is operational (screen, speakers, headphones and remote) | Y |  |  |
| Operational | A/V inputs |  |  |  |
| Verify | Opening/closing sliding doors does not scratch the outside panels/fuel cap/inside of the door |  |  |  |
| Verify | Pre-tensioners on the seatbelts |  |  |  |
| Verify | Belt assembly to be correct (proper location and orientation of the clip) |  |  |  |
| Verify | Carpet cutout for spare tire winch (FWD only) |  |  |  |

### Third Row

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Undamaged | Seat upholstery | Y |  | Check no fabric/leather creases or other anomalies |
| Undamaged | Carpet, lower panels | Y |  |  |
| Undamaged | Floor mat | Y |  |  |
| Alignment | Panel seams | Y |  |  |
| Operational | Pre-tensioners on the seatbelts | Y |  |  |
| Operational | Seat belt extension, retraction and overhead storage | Y |  |  |
| Operational | Belt assembly to be correct (proper location and orientation of the clip) | Y |  |  |
| Operational | Sunshades |  |  |  |
| Operational | Overhead lights | Y |  |  |
| Operational | Overhead vent assemblies |  |  |  |
| Operational | Power seat stow if equipped | Y |  |  |
| Operational | Rear window vents |  |  |  |

### Cargo Area

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Undamaged | Headliner for sagging along full length |  |  |  |
| Undamaged | Carpet and side panels | Y |  |  |
| Undamaged | Inside rear door panel | Y |  |  |
| Operational | Cargo area light | Y |  |  |
| Operational | Fold/unfold power third row seats | Y |  |  |
| Operational | 12 V DC outlet |  |  |  |
| Operational | 120 V AC outlet |  |  |  |
| Installed | Vehicle jack and tire iron | Y |  |  |
| Verify | Temporary license is not on defroster grid | Y |  |  |

## EXTERIOR ACCESSORIES

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Installed | Alloy Wheels, correct style and size |  |  |  |
| Installed | Bike attachment |  |  |  |
| Installed | Door edge guards |  |  |  |
| Installed | Dual port receiver hitch and wiring harness |  |  |  |
| Installed | Exhaust tip |  |  |  |
| Installed | Gold emblem package |  |  |  |
| Installed | Hood protector/deflector |  |  |  |
| Installed | Sunnroof wind deflector |  |  |  |
| Installed | Mud guards - 4X |  |  |  |
| Installed | Premium license plate frame |  |  |  |
| Installed | Rear spoiler |  |  |  |
| Installed | Roof rack attachment adapters |  |  |  |
| Installed | Running boards |  |  |  |
| Installed | Ski attachment |  |  |  |
| Installed | Stripes |  |  |  |
| Installed | Undercoating/protectant package |  |  |  |
| Installed | Wheel locks |  |  |  |
| Installed | Bodyside and/or Chrome Accent Moulding | Y |  |  |
| Installed | Paint protection film |  |  |  |

## INTERIOR ACCESSORIES

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Installed | Ashtray kit with lighter |  |  |  |
| Installed | All-weather mats |  |  |  |
| Installed | Auto-dimming mirror(s) |  |  |  |
| Installed | Cargo Logic Tote |  |  |  |
| Installed | Cargo nets |  |  |  |
| Installed | Carpet floor mats |  |  |  |
| Installed | Emergency assistance kit |  |  |  |
| Installed | First aid kit |  |  |  |
| Installed | Front window tint |  |  |  |
| Installed | Light package |  |  |  |
| Installed | Power lumbar support upgrade |  |  |  |
| Installed | Simulated woodgrains |  |  |  |
| Installed | Threshold/door sill protectors |  |  |  |
| Installed | VIP glass breakage sensor |  |  |  |
| Installed | Wireless headphones |  |  |  |
| Installed |  |  |  |  |

## TEST DRIVE (before final paper signing)

| Item | OK? | Notes |
| --- | --- | --- |
| Remote Engine Start (if equipped) and STOP |  |  |
| Engine start |  |  |
| Warning lights, gauge operation |  |  |
| Brake and parking brake performance |  |  |
| Engine performance, idle speed |  |  |
| Exhaust smoke |  |  |
| Transmission performance |  |  |
| Steering performance |  |  |
| Suspension performance |  |  |
| Any abnormal noise or vibration |  |  |
| Any squeaks or rattles |  |  |
| Any abnormal wind noise |  |  |
| Pull to the left/right under braking |  |  |
| Air conditioner/ air flow servo buzz |  | Recent complaints of a periodic buzz, similar to CD changing sound, that last for a second or two, and can happen once every minute or two. Happens mostly under AUTO mode and warm engine. |
| Acceleration |  |  |
| Turning |  |  |
| Compass operation |  |  |
| Navigation operation |  |  |
| Cruise control |  |  |
| (Radar) cruise control (if equipped) |  |  |
| Parking sonar |  |  |
| Heater |  |  |
| Air conditioning |  |  |
| Test Blind Spot Monitoring |  |  |
| Test Rear Cross Monitoring |  |  |
| Test VDIM – if you can figure out how! |  |  |
| Engine stop |  |  |

## LOOSE ITEMS THAT MAY COME WITH YOUR VAN

| Item | OK? | Notes |
| --- | --- | --- |
| 2 wireless headphones |  |  |
| 2 key fob transmitters |  |  |
| 2 main keys and 1 valet key as equipped |  |  |
| 4 body plugs |  | Should be installed during pre-delivery service |
| Center console box tray cup holder and small tray |  | Inside center console |
| Floor mats |  | If purchased |
| Front license plate bracket |  | Cargo area, if not installed |
| Key number plate |  |  |
| Navigation DVD or SD and case |  |  |
| Owner’s Manual - Car |  |  |
| Owner’s Manual - Navigation |  |  |
| Owner’s Manual - Radio |  |  |
| Owner’s Manual - RSE |  |  |
| Quick Start Guide |  |  |
| RSE Blu-ray/DVD remote control |  |  |
| Sealant cleaner |  | Inside lower glove box |
| Trailer hitch ball mount if equipped |  | Cargo area |
| Towing receiver protective cap if equipped |  | Installed on receiver |
| Warranty booklet - Tires |  |  |
| Warranty booklet - Vehicle |  |  |