# 2009 Venza Delivery Checklist – Version 1.05

## What to bring with you

- Tire pressure gauge

- 1-6 CDs with your favorite music (depending on the size of your CD-changer)

- DVD with your favorite movie (if you have rear seat entertainment (RSE))

- Car cell phone charger (to test 12V outlets)

- Low-power 110V device (100 watts max) with 2-prong plug (to test power inverter, part of RSE)

- Hi intensity flash light (peeking in the dark, testing auto rear view mirror, testing auto hi-beam dimmer)

- Insurance card and driver’s license

- Some way to pay

- Someone to drive the other vehicle home if not trading-in

## If Trading-In

- Title for the trade-in car (and all other required documents)

- All sets of keys + valet key, all key fob transmitters

- User Manual(s)

- PIN codes for keys, car stereo, Navigation system

- If Navigation on the trade-in, reset personal addresses and previous destinations

- If OnStar, XM or Sirius equipped, stop the service or transfer to new vehicle

- Reset Homelink remote control, if equipped

- Remove all personal belongings, check behind sun visors, under seats, storage compartments, plate frames etc.

## Exterior Initial Walk Around

- Walk once around looking at the overall condition

- Remote lock/panic/unlock using each key fob remote

- Pull the hood release lever (under the dash)

- Perform under hood inspection first (see table below) then start engine and listen for noises while outside the vehicle, with and without A/C, audio system off

- Turn on headlights, fog lights and emergency blinkers

### Under hood

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
|  | Open the hood | Y |  |  |
| Operational | Hood prop rod | Y |  |  |
| Undamaged | Battery, wiring, hoses and drive belts | Y |  |  |
| Level | Antifreeze | Y |  |  |
| Level | Windshield washer fluid | Y |  |  |
| Level | Brake fluid | Y |  |  |
| Level | Engine oil | Y |  |  |
| Installed | Spare fuses in fuse block (in top cover) | Y |  |  |
| Verify | No fluid leaks in engine compartment | Y |  |  |
| Operational | Close the hood | Y |  |  |

### Front

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Undamaged | Body panels for correct color, dents, scratches, spots, paint quality | Y |  |  |
| Undamaged | Front bumper fascia | Y |  |  |
| Undamaged | Windshield glass and frame | Y |  |  |
| Secure | Cowling (black plastic strip) is glued to the bottom of the windshield glass | Y |  |  |
| Alignment | Body panels and front fascia seams even | Y |  |  |
| Alignment | Hood with the body, head lights and front fascia | Y |  |  |
| Operational | Headlights, Fog lights, Side Marker Lights | Y |  |  |
| Secure | Wiper arms and blade inserts (heated, if installed) |  |  |  |
| Installed | Front license plate bracket (if required) |  |  |  |
| Verify | Windshield glass VIN etch (if ordered) | Y |  |  |
| Verify | Any required state inspection sticker(s) | Y |  | Current month, correct year? |

### Driver's Side

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Undamaged | Body panels for dents, scratches, spots, paint quality | Y |  |  |
| Undamaged | Door seals | Y |  |  |
| Undamaged | Side Mirror surface | Y |  |  |
| Undamaged | Window glass and window seals | Y |  |  |
| Undamaged | Wheel rims, tires, valve stem caps, 32 psi +/- 2 | Y |  |  |
| Undamaged,Secure | Running boards (if installed) | Y |  |  |
| Secure | Mud flaps (if installed) | Y |  |  |
| Alignment | Body panel seams even | Y |  |  |
| Operational | Fold/unfold manual mirror | Y |  |  |
| Operational | Front and rear door courtesy light operates, both doors open and close normally | Y |  |  |
| Verify | Window glass VIN etch (if required) | Y |  |  |
| Secure | Rear Wheel-well lining fasteners – tighten | Y |  | Build problem |
| Operational | Gas cap can be stowed on fuel filler door, nominal remove and replace on filler (one click only please) | Y |  |  |

### Rear

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Undamaged | Rear door for dents, scratches, spots, paint quality | Y |  |  |
| Undamaged | Rear bumper fascia | Y |  |  |
| Undamaged | Door seals |  |  |  |
| Undamaged | Window glass | Y |  |  |
| Secure | Rear spoiler | Y |  |  |
| Secure | Rear wiper arm and blade insert |  |  |  |
| Alignment | Rear door seams even | Y |  |  |
| Operational | Tail and license plate lights | Y |  |  |
| Operational | Open/close power rear door by key remote | Y |  |  |
| Operational | Open/close power rear door by rear handle | Y |  |  |
| Operational | Power rear door reverses direction when there is an obstruction | Y |  |  |
| Operational | Open driver's door and disable power rear door | Y |  | Knee level, left edge |
| Operational | Power rear door does not open electrically | Y |  |  |
| Operational | Open/close power rear door manually | Y |  |  |
| Operational | Enable power rear door and open | Y |  |  |
| Alignment | Radio Antenna (square to body) | Y |  |  |
| Installed | Towing receiver hitch cap and wiring(if installed) |  |  |  |
| Secure | Exhaust pipe Chrome Tips tight | Y |  | Build Problem (1-I4 / 2-V6) |
| Verify | Window glass VIN etch (if required) | Y |  |  |

### Passenger Side

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Undamaged | Body panels for dents, scratches, spots, paint quality | Y |  |  |
| Undamaged | Door seals | Y |  |  |
| Undamaged | Side Mirror surface | Y |  |  |
| Undamaged | Window glass and window seals | Y |  |  |
| Undamaged | Wheel rims, tires, valve stem caps, 32 psi +/- 2 | Y |  | Should be +/- 0.5 |
| Undamaged,Secure | Running boards (if installed) | Y |  |  |
| Secure | Mud flaps (if installed) | Y |  |  |
| Alignment | Body panel seams even | Y |  |  |
| Operational | Fold/unfold manual mirror | Y |  |  |
| Operational | Front and rear door courtesy light operates, both doors open and close normally | Y |  |  |
| Secure | Rear Wheel-well lining fasteners – tighten | Y |  | Build problem |
| Verify | Window glass VIN etch (if required) | Y |  |  |

### Top (open doors and stand on threshold)

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Undamaged | Roof panel for dents, scratches, spots, paint quality | Y |  |  |
| Alignment | Roof line to Spoiler | Y |  |  |
| Undamaged | Sunroof glass and seal (if ordered) | Y |  |  |
| Undamaged | Moonroof glass and seal (if ordered) | Y |  |  |
| Secure | Roof rack rails and crossbars (if installed) | Y |  |  |

### Underneath

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Undamaged | CV joints, drive shafts | Y |  |  |
| Undamaged | Exhaust system | Y |  |  |
| Undamaged | Shocks and springs | Y |  |  |
| Undamaged | Ball joints, tie rods, bushings | Y |  |  |
| Undamaged | Brakes | Y |  |  |
| Installed | Rubber body plugs installed (2) |  |  |  |
| Installed | Spare tire and correct pressure | Y |  |  |
| Verify | No fluid leakage | Y |  |  |
| Verify | Nothing dangling | Y |  |  |

## Interior

### Driver Seat

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Undamaged | Seat upholstery, correct color, look, feel, seams | Y |  |  |
| Undamaged | Carpet, lower panels | Y |  |  |
| Undamaged | Floor mat and both mat hooks | Y |  |  |
| Undamaged | Windshield glass optical defects | Y |  |  |
| Undamaged | Inside Door panel | Y |  |  |
| Alignment | Dash and door panel seams | Y |  |  |
| Operational | Seat power adjustment, recline, lumbar (as ordered) | Y |  |  |
| Operational | Seat heat (if installed) |  |  |  |
| Operational | Seat belt extension and retraction | Y |  |  |
| Operational | Overhead console sunroof lift/open/full open/close | Y |  |  |
| Operational | Overhead console controls | Y |  |  |
| Operational | Master light switch turns off all interior lights | Y |  |  |
| Operational | Steering wheel tilt/telescope adjustments | Y |  |  |
| Operational | Horn, instrument panel, gauges | Y |  |  |
| Operational | Turn signals – rate OK, if fast bulb out | Y |  |  |
| Operational | Front/rear windshield washers, heat if installed |  |  |  |
| Operational | A/C controls (both zones) |  |  |  |
| Operational | Radio (AM/FM/SAT), including steering wheel-mounted controls (as ordered) |  |  |  |
| Operational | AM/FM/Sat Radio Reception and audio controls |  |  |  |
| Operational | Open/close for access to CD changer |  |  |  |
| Operational | CD changer loads/unloads disks and plays them |  |  |  |
| Operational, Aligned | Rearview camera, good color, good focus, good alignment |  |  |  |
| Operational | Navigation voice guidance (a few simple commands) |  |  |  |
| Operational | Navigation voice commands (one command) |  |  |  |
| Operational | Sun visor mirror lights |  |  |  |
| Operational | All dash lights |  |  |  |
| Operational | Ignition button light (if equipped) |  |  |  |
| Operational | Map lights/dome lights |  |  |  |
| Operational | Door courtesy light, manual mode |  |  |  |
| Operational | 12VDC outlets – three total |  |  |  |
| Operational | Side mirror (left/right) control left/right/up/down |  |  |  |
| Operational | Automatic headlights (cover dash sensor in daytime) |  |  |  |
| Operational | Flash high beams, test auto Hi-Beam dimmer | Y |  |  |
| Operational | HID level adjust |  |  |  |
| Operational | Auto down/up all windows |  |  |  |
| Operational | Power door lock/unlock from manual control |  |  |  |
| Operational | Heated mirrors |  |  |  |
| Operational | Start engine with all (smart) keys, individually | Y |  |  |
| Operational | Bluetooth connectivity |  |  |  |
| Operational | Cup holder lighting (rubber insert out, headlights on) |  |  |  |
| Verify | Cup holder rubber insert (1 - sigh) |  |  |  |
| Alignment | Center dash console bins |  |  |  |
| Installed | Center console |  |  |  |
| Installed | Shift-lock override button cover |  |  |  |
| Verify | Latest navigation DVD version ?? (eject disk from head unit - the version is on the disk's label). |  |  | Set proper region set before testing – Page 52 Nav Manual |
| Verify | Full tank of gas |  |  |  |
| Verify | Valet key will open/close door and start the car |  |  | If not smartkey |
| Verify | In the dark, make sure all buttons illuminate |  |  | All very dim, requires night |
| Verify | Navigation system stored destinations empty – use “Delete All” to clear any present |  |  |  |
| Verify | While all doors and windows are closed, open and close the driver's door, make sure no excessive force is required to close it (air tightness of the car) |  |  |  |
| Verify | Pre-tensioners on the seatbelts | Y |  |  |
| Verify | Belt assembly to be correct (proper location and orientation of the clip) | Y |  |  |
| Verify | Clock is set |  |  |  |
| Verify | Custom body electronics system programming requested was done correctly |  |  |  |
| Verify | Odometer has few nominal miles | Y |  | Reduce price for demo |

### Passenger Seat

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Undamaged & Color Correct | Seat upholstery | Y |  |  |
| Undamaged | Carpet, door panel | Y |  |  |
| Undamaged | Floor mat | Y |  |  |
| Undamaged | Windshield glass optical defects | Y |  |  |
| Undamaged | Dash and door panel seams | Y |  |  |
| Alignment | Panel seams | Y |  |  |
| Alignment | Glove compartment | Y |  |  |
| Operational | Power door lock/unlock | Y |  |  |
| Operational | Power window | Y |  |  |
| Operational | Sun visor lights | Y |  |  |
| Operational | Seat power adjustment, recline, lumbar (as ordered) | Y |  |  |
| Operational | Seat heat (if installed) | Y |  |  |
| Operational | Seat belt extension and retraction |  |  |  |
| Verify | Pre-tensioners on the seatbelts |  |  |  |
| Verify | Belt assembly to be correct (proper location and orientation of the clip) |  |  |  |

### Second Row

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Undamaged | Seat upholstery | Y |  |  |
| Undamaged | Carpet, door panels | Y |  |  |
| Undamaged | Floor mat | Y |  |  |
| Undamaged | Pockets on the back of the front seats | Y |  |  |
| Undamaged | Door panel seams | Y |  |  |
| Alignment | Panel seams | Y |  |  |
| Operational | Door lock/unlock | Y |  |  |
| Operational | Child safety locks | Y |  |  |
| Operational | Power windows | Y |  |  |
| Operational | Each section of rear seat goes forward flat | Y |  |  |
| Operational | Each section of rear seat reclines | Y |  |  |
| Operational | Seat belt extension and retraction | Y |  |  |
| Operational | Overhead lights | Y |  |  |
| Operational | Console vent assemblies |  |  |  |
| Operational | Arm rest, cup holder cover, cup holders | Y |  |  |
| Operational | Insert DVD into the RSE disk reader and start the movie (if installed) | Y |  |  |
| Operational | RSE is operational (screen, speakers, headphones and remote) (if installed) | Y |  |  |
| Operational | A/V inputs (if installed) |  |  |  |
| Verify | Pre-tensioners on the seatbelts |  |  |  |
| Verify | Belt assembly proper location and clip orientation |  |  |  |

### Cargo Area

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Undamaged | Headliner for sagging along full length |  |  |  |
| Undamaged | Carpet and side panels | Y |  |  |
| Undamaged | Inside rear door panel | Y |  |  |
| Operational | Cargo area light | Y |  |  |
| Operational | Release handles operate second row seats | Y |  |  |
| Operational | 12VDC outlet / 110 VAC inverter (if RSE) |  |  |  |
| Installed | Compact Spare, Vehicle jack and tire iron | Y |  |  |
| Verify | Temporary license is not on defroster grid | Y |  |  |

## Exterior Accessories

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Installed | Alloy Wheels, clean, bright, unscratched |  |  |  |
| Installed | Bike or other carrier attachment (if ordered) |  |  |  |
| Installed | Door edge guards (if ordered) |  |  |  |
| Installed | Dual port receiver hitch and wiring harness |  |  |  |
| Installed | Hood protector |  |  |  |
| Installed | Premium license plate frame |  |  |  |
| Installed | Running boards |  |  |  |
| Installed | Stripes (if ordered) |  |  |  |
| Installed | Undercoating/protectant package (if ordered) |  |  |  |
| Installed | Wheel locks (if ordered) |  |  |  |
| Installed |  |  |  | What Else Did You Order? |

## Interior Accessories

| Check | Item | At Dealer | OK? | Notes |
| --- | --- | --- | --- | --- |
| Installed | Ashtray kit with lighter |  |  |  |
| Installed | Carpet floor mats – 5 mats |  |  |  |
| Installed | All-weather mats vice carpeted mats |  |  |  |
| Installed | Auto-dimming mirror (if installed) |  |  |  |
| Installed | Cargo Logic Tote |  |  |  |
| Installed | Cargo nets |  |  |  |
| Installed | Emergency assistance kit (if ordered) |  |  |  |
| Installed | First aid kit (if ordered) |  |  |  |
| Installed | Front window tint |  |  |  |
| Installed | Simulated wood trims, appearance |  |  |  |
| Installed | Threshold/door sill protectors |  |  |  |
| Installed | VIP glass breakage sensor (if ordered) |  |  | Test – arm, open hand slap |
| Installed | Wireless headphones (if ordered) |  |  |  |
| Installed |  |  |  | What Else Did You Order? |

## Test Drive (before final paper signing)

| Item | OK? | Notes |
| --- | --- | --- |
| Engine start |  |  |
| Warning lights, gauge operation |  |  |
| Brake and parking brake performance |  | Under break-in conditions please, Manual Page 154 |
| Engine performance, idle speed |  |  |
| Exhaust smoke |  |  |
| Transmission performance (auto and manual) |  |  |
| Steering performance (lock to lock moving slow) |  |  |
| Suspension performance |  |  |
| Any abnormal noise or vibration |  |  |
| Any squeaks or rattles |  |  |
| Any abnormal wind noise |  |  |
| Pull to the left/right (acceleration, cruise, braking) |  |  |
| Acceleration |  | Under break-in conditions please, Manual Page 154 |
| Turning |  |  |
| Compass operation – check for proper deviation |  | Manual Page 420 |
| Navigation operation |  |  |
| Cruise control |  |  |
| AM/FM(/SAT if ordered) operational |  | Tune SAT channel 5, hear 1950's music to confirm demo start |
| Air conditioning and Heater |  |  |
| Engine stop |  |  |

## Loose items that may come with the van

| Item | OK? | Notes |
| --- | --- | --- |
| 2 wireless headphones |  |  |
| 2 key fob transmitters |  |  |
| 2 main keys and 1 valet key (if not smartkey) |  |  |
| 2 body plugs |  | Should be installed during pre-delivery service |
| Front license plate bracket |  | Cargo area, if not installed |
| Key number plate |  |  |
| Navigation DVD and case |  |  |
| Owner’s Manual – Car (also a Quick Start Guide) |  |  |
| Owner’s Manual - Navigation |  | 300 pages of goodness |
| Owner’s Manual - Radio |  | 600 pages of goodness |
| Owner’s Manual - RSE |  |  |
| RSE DVD remote control |  |  |
| Sealant cleaner (if ordered) |  | Inside glove box |
| Trailer hitch ball mount (if ordered) |  | Cargo area |
| Towing receiver protective cap |  | Installed on receiver |
| Warranty booklet - Tires |  |  |
| Warranty booklet - Vehicle |  |  |

Edited from a Sienna Checklist by Steve Mahler and the ToyotaNation.Com - 2009